Tutorial on Adding SEM Sensors Based on MQTT in Home Assistant

First，turn on the switch in the Home Assistant app.

Connect and configure MQTT

Setting--> Devices & Services --> Integrations --> Add Integration --> Search for MQTT --> Click to configure information such as domain names and ports.

mqtt broker:    mqtt001.tumblevd.com

Port:          6000

Username:     your email

Password:      your password(If it's a quick login via Apple or Google and no password has been set, the default password is "semmeter"）

Modify the configuration.yaml

Copy the configuration code to configure.yaml and replace all the MAC addresses (the user's MAC address will be shown during the network configuration).

Display semmeter data (Developer tools -> states)

Add CARD

Connect to your own MQTT server.
1：Set the IP address, port, account number and password of your MQTT server for the SEMMETER device.

2：Configure your Home Assistant to connect to your server.

configuration.yaml mqtt demo code:

# Demo mac DCDA0C826ABD   {Your own MAC address and the name of the Bluetooth network configuration will be displayed.}

mqtt:

sensor:

- name: "MAIN current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[16][1] | float | multiply(0.01) +  value_json.sense[17][1] | float | multiply(0.01) + value_json.sense[18][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "MAIN active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[16][2] | float | multiply(0.01) + value_json.sense[17][2] | float | multiply(0.01) + value_json.sense[18][2] | float | multiply(0.01)}}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "MAIN energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[16][3] | float | multiply(0.001) + value_json.sense[17][3] | float | multiply(0.001) +value_json.sense[18][3] | float | multiply(0.001)}}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "MAIN energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[16][4] | float | multiply(0.001) +value_json.sense[17][4] | float | multiply(0.001) + value_json.sense[17][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "A channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[16][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "A channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[16][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "A channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[16][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "A channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[16][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "A channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[16][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "B channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[17][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "B channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[17][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "B channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[17][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "B channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[17][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "B channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[17][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "C channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[18][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "C channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[18][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "C channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[18][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "C channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[18][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "C channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[18][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "1 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[0][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "1 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[0][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "1 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[0][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "1 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[0][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "1 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[0][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "2 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[1][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "2 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[1][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "2 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[1][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "2 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[1][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "2 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[1][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "3 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[2][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "3 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[2][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "3 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[2][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "3 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[2][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "3 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[2][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "4 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[3][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "4 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[3][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "4 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[3][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "4 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[3][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "4 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[3][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "5 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[4][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "5 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[4][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "5 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[4][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "5 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[4][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "5 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[4][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "6 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[5][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "6 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[5][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "6 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[5][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "6 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[5][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "6 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[5][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "7 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[6][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "7 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[6][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "7 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[6][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "7 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[6][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "7 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[6][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "8 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[7][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "8 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[7][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "8 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[7][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "8 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[7][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "8 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[7][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "9 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[8][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "9 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[8][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "9 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[8][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "9 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[8][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "9 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[8][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "10 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[9][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "10 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[9][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "10 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[9][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "10 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[9][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "10 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[9][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "11 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[10][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "11 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[10][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "11 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[10][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "11 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[10][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "11 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[10][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "12 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[11][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "12 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[11][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "12 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[11][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "12 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[11][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "12 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[11][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "13 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[12][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "13 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[12][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "13 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[12][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "13 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[12][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "13 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[12][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "14 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[13][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "14 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[13][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "14 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[13][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "14 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[13][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "14 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[13][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "15 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[14][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "15 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[14][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "15 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[14][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "15 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[14][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "15 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[14][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "16 channel voltage"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[15][0] | float | multiply(0.1) }}"

unit_of_measurement: "V"

icon: "mdi:flash"

- name: "16 channel current"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[15][1] | float | multiply(0.01) }}"

unit_of_measurement: "A"

icon: "mdi:current-ac"

- name: "16 channel active power"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[15][2] | float | multiply(0.01) }}"

unit_of_measurement: "W"

icon: "mdi:power-plug"

- name: "16 channel energyIn"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[15][3] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing

- name: "16 channel energyOut"

state_topic: "SEMMETER/DCDA0C826ABD/HA"

value_template: "{{ value_json.sense[15][4] | float | multiply(0.001) }}"

unit_of_measurement: "kWh"

icon: "mdi:counter"

device_class: energy

state_class: total_increasing